| Use Case Description | - Create a zone to host a K8s cluster within a composable DC fabric |
| --- | --- |
| Actors | - Fabric Manager, Resource Manager, Composer, Administrator |
| Description | - Use Redfish Composition to allocate a zone object to define a virtual, private network within a larger fabric |
| Comments | - Ethernet network available for TCP, UDP, SCTP |
| Input Data | - Current Redfish Composition Resource Zones - list of cluster members defined - list of free non-associated members - members that are able to be in multi-zone membership - cluster members security restrictions |
| Preconditions | - Ethernet fabric interfaces state up - Ethernet fabric enabled - Interface MAC addresses - DHCP IP range |
| Postconditions | - Resource zone complete with sane membership connections - List of Zone membership - Zone creation success - MAC address list available for DHCP server |
| Trigger | - Fabric Manager request to create a zone with a certain quantity of members - OFMF parse a Create Zone HTTP request |
| Normal Flow | - Parse the list of cluster members - Query list of non-associated members - Query list of multi-zone capable members - DHCP range of valid addresses - Query list of member’s security-level membership - Using Redfish Composer, create a fabric zone object - Fill in the resources to be contained in the zone - Make symmetric connections? - Read member interfaces - Calculate a new route table - Using Redfish Composer, update the Zone membership - Report Composer Zone success - Update list of associated members - Update list of non-associated members - Post the Redfish zone object to the OFMF’s resource tree - Respond to client with success - Respond to client with Zone Object members - MAC address list available for DHCP server |
| Alternate Flow 1 |  |