| # | Feature | Description |  | Comment | Priority |
| --- | --- | --- | --- | --- | --- |
| 0 | General Requirements |  |  |  |  |
| 0.1 | Byte granular access |  | Generic byte granular interface which handles access to both DRAM and other types of byte addressable storage |  | MH (must have) |
| 0.2 | RDMA Read, Write, Send, Receive, atomics |  | Support all flavors of RDMA access semantics | May be limited per endpoint or device See 19, 43, 65 | R/W/S/R MH Atomic HD (Highly desirable) |
| 0.3 | Expose NVM specifics |  | Expose (and make use of) memory type specific access behavior (access latency, staged WRITE, persistency, …) |  | MH |
| 0.4 | Local and remote storage access |  | Access locally attached IO memory the same way as networked NVM or DRAM | See also 6.1 | HD |
| 0.5 | Application private access |  | Both privileged (OS level) and protected application level private end points |  | MH |
| 1 | Work Request Execution Ordering |  |  |  |  |
| 1.1 | Strict Ordering |  | Strictly execute in order | See 55, 156 | MH |
| 1.2 | Lazy Ordering |  | Execute in any order/parallel but may signal completion in order posted | Efficient See 23 | OP (optional) |
| 1.3 | Explicit Unordered |  | Execute in any order/parallel but signal effective completion order | Even more efficient See 55 | OP |
| 1.4 | Ordering Selectable |  | Strict/non strict per WR, Explicit/Lazy per EP or device | See 55 | OP |
| 1.5 | Fencing |  | Barrier: Halt execution of successive WR’s until this done, execute all previous before | Per WR See 140 | HD |
| 1.6 | Fusing |  | Execute 2 successive WR’s in order, stop on first failure | E.g.: Atomic Compare + READ/WRITE, READ + TRIM, | OP |
| 2 | Data Placement Completion Level (per WR, for all Send, Receive, READ, WRITE) |  |  |  |  |
| 2.1 | Data Placement with lazy completion |  | Data have completely reached target RDMA device but may have not yet been placed into persistence domain, if NVM | May be lost if power outage See 168 | MH |
| 2.2 | Data Placement committed |  | Data have been completely placed into persistence domain, if NVM; Depending on device and chipset capabilities (ADR, auto read-back), different semantics could be adopted by the provider to ensure data commitment | Data in persistent store if NVM See 168 | HD |
| 3 | Read and Write Acceleration |  |  |  |  |
| 3.1 | Read Ahead |  | Explicit hint to pre-fetch data for successive READ’s | Per WR | OP |
| 3.2 | Write More |  | Explicit hint successive WRITEs are to be expected | Per WR | OP |
| 4 | Memory Registration and Addressing |  |  |  |  |
| 4.1 | Zero Based Addressing |  | Support zero based addressing for memory reference in WR | See 4.2 vs. 4.3 and 4.4 See 67, 88 | HD |
| 4.2 | Registration by mapped VA |  | Allow NVM registration using VA from resource mapping |  | MH |
| 4.3 | Registration by opaque resource ID |  | Allow registration of memory resource by provider interpreted opaque handle | e.g. file name, path, requires ZBA | HD |
| 4.4 | Re-registration by reservation key |  | Allow re-registration of previously registered persistent memory object | e.g., resume reservation after reboot | HD |
| 4.5 | Resize memory registration |  | Shrink or grow memory reservation while maintaining same reservation key | See 127 | HD |
| 4.6 | Fast registration and invalidate STag |  | Allow Fast registration and send with invalidate Stag semantics at API | Exist today | MH |
| 4.7 | Physically discontinuous buffer |  | Application should not care about underlying physical contiguity of allocated memory | See 60 | MH |
| 5 | NVM Specific Commands Support |  |  |  |  |
| 5.1 | TRIM support - release |  | Inline TRIM support per reservation key with offset and length, releases resources, but doesn’t guarantee obliteration of the memory | Implies fencing | OP |
| 5.2 | TRIM support - obliterate |  | Obliterates the memory |  | OP |
| 6 | Accessing storage |  |  |  |  |
| 6.1 | Single EP for operations on local storage |  | Single EP to connect to local service to access local NVM, no explicit peer endpoint to be set up | Embed ‘local peer’ as OS service | HD |
| 6.2 | Single sided RMA |  | Explicit endpoint creation on active side only, memory server exports access to its memory | No connection management at memory server See 4, 11, 118, 120 | OP |
| 7 | Mix DRAM and NVM access |  |  |  |  |
| 7.1 | WR may reference any memory type |  | Per WR, referenced communication buffers might be of any memory type (requires single local key space) | Only if provider supported See 79 | HD |
| 7.2 | SGL may reference a mix of storage and DRAM |  | In a single WR, individual SGE’s may reference different types of storage (requires single local key space) | Only if provider supported | HD |
| 8 | RDMA Transport |  |  |  |  |
| 8.1 | Multi-destination RMA operation |  | One single IO operation targets multiple memory locations on multiple remote nodes | Need reliable multicast See 99, 147 | OP |
| 8.2 | Transparent multi-path |  | Aggregate multi-port/link/NIC/HCA under single endpoint, transparent failover, in-order delivery, fencing support |  | OP |
| 9 | End Point |  |  |  |  |
| 9.1 | Flow Control |  | Options to configure flow-control at end-point level | See 63, 160 | OP |
| 10 | Completion Notification |  |  |  |  |
| 10.1 | Remote notification |  | In RDMA write / read operations, provide the option of generating a completion event at the remote end | Exist today (e.g., write with immediate) See 167 | MH |